11 NCAC 08 .1114
INSULATION AND VENTILATION

(a)  The home inspector shall inspect:

(1)
Insulation and vapor retarders in unfinished spaces;

(2)
Ventilation of attics and foundation areas;

(3)
Kitchen, bathroom, and laundry venting systems; and

(4)
The operation of any readily accessible attic ventilation fan, and, when temperature permits, the operation of any readily accessible thermostatic control.

(b)  The home inspector shall describe:

(1)
Insulation in unfinished spaces; and

(2)
The absence of insulation in unfinished space at conditioned surfaces.

(c)  The home inspector is not required to report on:

(1)
Concealed insulation and vapor retarders; or

(2)
Venting equipment for household appliances that are not required to be inspected pursuant to the North Carolina Home Inspector Standards of Practice.

(d)  The home inspector shall:

(1)
Move insulation where readily visible evidence indicates a problem; and

(2)
Move floor insulation where plumbing drain/waste pipes penetrate floors, adjacent to earth-filled stoops or porches, and at exterior doors.

History Note:
Authority G.S. 143-151.49;

Codifier determined that agency findings did not meet criteria for temporary rule Eff. October 15, 1996;

Temporary Adoption Eff. October 24, 1996;

Eff. July 1, 1998;

Amended Eff. February 1, 2007; April 1, 2005; May 1, 2003;

Readopted Eff. October 1, 2018.



NUMPAGES
1
 of 


